INFORMACION IMPORTANTE DE LOS SISTEMAS GPS DE BATERIA

Es muy importante entender cómo funcionan los equipos GPS de baterías para poder prolongar al máximo la duración de su batería ya que son MUY DIFERENTES a los GPS que están conectados todo el tiempo a la alimentación como los automotrices.

Diferencias entre Rastreo por SMS o Plataforma:

SMS: El rastreo del GPS se hace a través de llamadas o SMS. En este tipo de rastreo la información y/o ubicación que recibe es totalmente actualizada y en tiempo real. No es necesario tener un plan de datos, solo tener saldo para que el equipo pueda contestarle el SMS, y solamente enviara la ubicación cuando usted se lo solicite mediante una llamada o SMS

PLATAFORMA: El equipo enviará la ubicación mediante datos que envía al servidor en internet a intervalos predefinidos de tiempo y cuando usted desee consultar la ubicación entrará a la plataforma y ahí le aparecerá la última ubicación que el equipo le envió datos. Usted definirá los intervalos de tiempo en los que el equipo estará enviando datos a la plataforma. Para este tipo de modalidad requiere tener un plan de datos y la configuración del servidor o plataforma que van a utilizar para enviarle los datos.
Generalmente los GPS ya traen configurado el servidor de fábrica aunque en el manual están los datos para cambiar de servidor/plataforma.
Los equipos consumen menos de 10mb por mes ya que lo que la información enviada al servidor/plataforma es muy poca.

Intervalos de Actualización en la Plataforma y consumo de Datos:

Cada que el equipo envía la ubicación consume energía ya sea si envía SMS o si envía Datos a la plataforma. Es por ello que los equipos que van conectados a una fuente de alimentación como los GPS automotrices pueden enviar datos cada 10 segundos si es necesario y con ello se logra un rastreo muy preciso en la plataforma ya que la última posición que vera es de hace 10 segundos.

Pero en los equipo que tienen pila la ubicación se envía a intervalos mucho mayores para lograr maximizar la batería. Por ejemplo algunos equipos vienen configurados de fabrica para enviar la ubicación a la plataforma una vez al dia, tal vez la ultima vez que enviaron la ubicacion fue a la 1pm y si usted entra a la plataforma a las 8pm vera la ubicación de la 1pm. Sin embargo siempre puede consultar la ubicación en tiempo real si solicita la ubicación via SMS ya que los SMS no dependen de la plataforma.

“Sugerencia: es mejor no utilizar la plataforma en GPS portátiles ya que al enviar la ubicación a intervalos muy separados de tiempo no se tiene una lectura exacta. Sin embargo, si le solicita la ubicación via SMS tendrá la ubicación en tiempo real”

Si desea utilizar la plataforma en un equipo GPS de batería por lo menos le sugerimos utilice intervalos de actualización muy amplios como una hora por lo menos para que su equipo no se acabe la batería muy rápida, cada equipo tiene diferentes maneras de cambiar el rango de actualización de datos en la plataforma, y también algunos equipos tiene la opción de apagar los datos para asi ya no utilizar la plataforma para nada y solo usarlo con SMS

Sistemas de Prolongación de Batería:

Para prolongar aún más la vida de la batería algunos equipos de batería incorporan sistemas de ahorro de energía por ejemplo:

- Algunos equipos pueden apagar el GPS y solo enviar la ubicación vía triangulación de señal celular o modo LBS, pero este sistema hace que la ubicación sea sumamente imprecisa, dando rango de error de hasta 100 metros.

- Algunos equipos incorporan sensores de movimiento asi que dejan de enviar señales si no detectan movimiento ya que dan por echo que no tiene caso enviar datos o ubicación si no se ha movido el GPS de el ultimo lugar donde enviaron la ubicación

- Algunos equipos tiene una modalidad que apagan totalmente el equipo y solo se despiertan para enviar la ubicación 1, 2, o 3 veces al dia. El resto del tiempo están totalmente apagados sin enviar datos o responder llamadas o SMS.

“Sugerencia: si desea usar la plataforma jamás utilice los modos de apagado ya que luego no lo va a poder sacar de ese modo fácilmente ya que todo el tiempo estará apagado despertando solo unos minutos al dia, lo que es mejor es definir un intervalo de envió a la plataforma muy amplio, de 10 minutos a una hora como mínimo”

Es un equipo con batería de alta duración, pero también diseñado pensando que solo se consulten pocas veces su ubicación para maximizar la vida de su batería por tanto incluye muchos sistemas que prolongan la vida de la misma

Por ejemplo, en uno de los modos si el equipo no detecta movimiento por un largo periodo se apagara, dejando de contestar las llamadas y de enviar la ubicación, y solo se volverá a encender si detecta movimiento, esto hace que a veces las personas piensen que se descompuso pero es que simplemente esta hibernando y no contestara ni hará nada hasta que se despierte.  Es por eso que es importante que lea cuidadosamente este manual.

Usted puede consultar la ubicación exacta en todo momento via SMS, o puede meterse a la plataforma y ver en donde estaba el equipo desde la ultima vez que le envio datos a la misma.

Si usted lo consultara una vez por dia o lo programara para que enviara la ubicación a la plataforma una vez por dia la pila le duraría hasta 3 años, sin embargo el GPS va programado en el MODO 1 el cual se diseño para enviar la ubicación una vez cada 3 minutos a la plataforma, haciendo que su pila solo le dure 10 dias

No importa que no consulte la ubicación via SMS o no entre a la plataforma, los GPS están enviando continuamente la ubicación a su plataforma en intervalos de tiempo, y esto es lo que afecta la duracion del mismo.

En los equipos que van conectados directamente a fuentes de poder como gps automotrices se envían los datos una vez cada 10 minutos y se logra un rastreo muy presiso, pero en este equipo la ubicación no puede ser tan rápida. 

De echo paa estos equipos recomendamos no usar la plataforma y mejor solo usarlo con sms únicamente cada que se desee saber donde esta el carro. Puede apagar la plataforma con el comando SMS123456 y la puede volver a prender con el comando GPRS123456

Este equipo puede funcionar en tres diferentes modalidades cada una pensando en ahorrar batería o en ser mas eficiente a costa de sacrificar la batería. Los modos son los siguientes:

MODOS DE FUNCIONAMIENTO

【 MODO 1 】: Rastreo en tiempo real (default)

Envia la ubicación a la plataforma 1 vez cada 3 minutos, logrando funcionar por tan solo 10 dias.

Se utilizara el comando asdfasdfadsfad

MODO 2:

7)  Working mode

【Mode 1】：real-time tracking (default)

every 3mins/1 times,can work 10 days,can’t anti-jammer and anti-detection

【Mode 2】：24 hours cycle mode

every 24 hours/1 times positioning,can work 3 years

【Mode 3】：clock mode(can set up 2-6 times positioning per day )

every 24 hours/2-6 times positioning,can work from 6 months to 18 months

【Mode 4】:  smart cycle positioning(10mins)

every 10 mins/1 times, device upload position , then device turn off

Until the next cycle positioning again

Positioning time interval can be changed, time is ≥10mins

【Mode 5】:  smart turn on/off mode (deep sleep save power mode)

(static turn off save power, vibration or falls off alarm start work)

NOTE: mode2、mode3、mode4 and mode5 is saving power mode

1)After device upload position to platform ,device will turn off automatic for save power)

2)During turn off,device not reply and offline platform is normal

3)device will online in 3 mintutes next cycle positioning,you can set device or change device mode at this period

Thank you for purchasing the tracker. This manual shows how to operate the device smoothly and correctly. Make sure to read this manual carefully before using this product. Please note that specification and information are subject to changes without prior notice in this manual. Any change will be integrated in the latest release. The manufacturer assumes no responsibility for any errors or omissions in this document.

1. Summary

Working Based on existing GSM/GPRS network and GPS satellites, this product can locate and monitor any remote targets by SMS or internet.

Specifications

| Content | Specs |
| --- | --- |
| Dim. | 93mm x 77mm x 27mm |
| Weight | 227g |
| Network | GSM/GPRS |
| Band | 850/900/1800/1900Mhz |
| GSM chip | MT6261 |
| GPS chip | Ublox 7020 |
| GPS sensitivity | -159dBm |
| GPS accuracy | 5m |
| Time To First Fix | Cold status 45s /Warm status 35s /Hot status 1s |
| Work voltage | 5V |
| Storage Temp. | -40°C to +85°C |
| Operation Temp. | -20°C to +55°C |
| battery | Chargeable 3.7V 6600mah Li-ion Battery Optional car charge:12-30V |
| Humidity | 5%--95% non-condensing |

# 2.Main functions

- GSM quad-band frequency;

- Real time tracking by SMS/GPRS

- Low battery alarm

- Movement alarm

- Into/out off Geo-fence alarm

- Add Vibration alarm

- Add Monitor

- Add Super-long standby 3 yeas (24-hour cycle positioning）

- Add strong magnet(free installation)

- Add Anti-jammer and Anti-detection function

- Add Anti-Disassemble and anti-Fall off alarm

- Add clock mode /fixed time positioning/smart cycle position

- add waterproof and dust proof

- add save power and GPRS

- Add AGPS function,receive GPS signals faster

- Add built-in storage memory

- Add app GPRS set

3.Led indicator status

| # LED INDICATOR STATUS | # MEANING |
| --- | --- |
| # Green LED Flashing | # Searching GSM signal |
| # Green LED continuously lit | # GSM signal OK |
| # Blue LED continuously Flashing per 3s every | # Searching gps signal |
| # Blue LED continuously lit | # gps signal ok |

3.1 Notices:

1).GPS tracking can be worked outdoor. Must have mobile phone gsm signal and have GPS signal,  It can block GPS satellite signal when there are metal or  building above the GPS tracker

2).Pls make sure GSM 2g SIM card or compatibility 2G GSM network sim card, supports making calls, sending messages, GPRS network.have money,Cancel the PIN code

- Quick using

Use mobile phone call the device and hang up

or sending message “smslink123456” to sim card in tracker ,then send message “time zone123456 time”tracker reply positioning information,time is loca time zone

Picture (1) have gps signal     Picture (2) no gps signal

Pic(1)：Replied real-time tracking information

Lat:22.67533 lon:114.03913  Positioning of the latitude and longitude values.

T:06/14/17 09:14  real-tracking time, the Default is London time. You can set a time zone

Google Link: This is the link to the Google map showing the position of the device.

- Pic( 2)：Replied last time position information. Now is LBS position

If you receive positioning information is picture 2, please go out the outdoor and Walk around with gps tracker,for receive The satellite signal

5:Support Website tracking or APP tracking check

If you need real-time tracking by APP ,please scan APP or Download “dagps”

5.1 Scan APP or Download APP

1)scan QR code---open with browser--- choose Android or Iphone---Download--  installation app

2) Download ios APP:login in app store---dagps---Install the dagps APP

5.2: login platform  positioning

1)Open www.gps110.org

Use tracker ID login, default password 123456.

PC login                        APP login

5.3 register Administrator account and tracking multiple devices

operation ：register APP accounts--- Fill in the APP---login in registered accounts vehicle list----add device---fill in ID--OK

Registration                                        fill in

Click vehicle list                            add device

If you add device more than two , continue to add ID number

If device online in platform, no need set apn commands

6.1 Set APN

SMS1: APN123456+space+Apn name

( APN name pls ask the sim card company ,for example tell you the apn name is ABC,so send :APN123456 ABC)

(If have the APN user name and password, need plus APN user name and password behind the APN name)

Eg: APN123456+space+APN name+space+ APN user name+space+   APNpassword

It is Real time positioning,every 3min upload position by default,you can change upload time interval

6.2. If use the other platform, start on line tracker

in advance Using mobile phone send SMS commands to device and set online platform (please set DNS and port,set apn)

(Use the default platform, don't need to operate 6.2)

8.Set Command by APP or SMS

NOTE:

1) Device default password:123456)

2)use of mobile phones APP or send message to the tracker about all set command

3)If you want standby time longer,  pls don't open move alarm, and real-time  location tracking.etc

4)Charging for 24 hours for the first time, device light not on

| NAME | COMMAND | - EXAMPLE |  |
| --- | --- | --- | --- |
| Set apn | apn123456 apn Name |  | apn123456 cmnet |
|  | apn123456 apn name user password |  | Apn123456 cmnet cmwap 66666 |
| Set IP /DNS port | adminip123456 IP port |  | Adminip123456 110.92.64.21 7018 |
|  | Adminip123456 DNS port |  | Adminip123456 www.gps110.org 7018 |

| INSTRUCTION | COMMAND | REPLY |
| --- | --- | --- |
| WHERE | smslink123456 | Get adress |
| Set admin number | admin123456 mobile number | admin ok |
| Cancel admin number | noadmin123456 mobile number | noadmin ok |
| Set time zone | time zone123456 8 | Time zone ok |
| Set time zone(half time zone) | Time zone123456 8 30 | time zone ok |
| set move alarm | move123456 (suggest not open for will not save power | move ok |
| Cancel move alarm | nomove123456 | nomove ok |
| Open low battery alarm | Lowbattery123456 on | Low batter ok |
| Cancel low battery alarm | Lowbattery123456 off | Low batter ok |
| Cancel GPRS | sms123456 | smsok |
| Set shock alarm | Shock123456 | Shock ok |
| Cancel shock alarm | noshock123456 | noshock ok |
| Listening to the sound | monitor123456 | Monitor ok |
| Cancel listening sound | tracker123456 | trackerok |
| Clock mode | clock123456 1200 1800 2100 (the time format is 24 hours system) | Clock ok |
| Smart turn on/off mode | static123456 | Static ok |
| Smart cycle mode(≥10mins) | Time123456 10 | time ok |
| real-time tracking (default) | t003m***n123456 | T n ok |
| Fall off alarm | Loosing123456 | Loosing ok |
| Check param | check123456 | param |
| Reset factory | reset123456 | reset ok |

| FAQ | Instructions/Solutions |
| --- | --- |
| Fail to turn it on | Please check device charged |
| No GSM signal | - Please check if SIM card installed correctly. - Please check if SIM card is 2G GSM network. - Don’t turn on the PIN code - no calls can be diverted - Please check if voltage of the power is normal - Please check if voltage of the power is normal 电源电压是否正常 |
| No GPS | A more open view of the sky is recommended to make sure the device can receive the GPS signal normally.in outdoor,or take device to walk around |
| No reply to SMS command | SMS commands password wrong or the format is wrong. restore factory settings send “begin123456” |
| No reply to calling & No alarm message | Authorized / admin phone number is incorrect or has not set up,check sim card is normal |
| SMS position OK, cannot use the APP and web platform | Pls check if APN is correctly set. And pls find out the correct APN content of the SIM card. |
| Platform position is difference from the actual position. | No uploading position time or no GPS signal,reply position is last times. If it shows like Pic 2, pls make sure the device can receive the GPS signal normally.in outdoor,and take device to walk around |