GSM/GPRS/GPS TRACKER

MANUAL

Preface

Thank you for purchasing the tracker. This manual shows how to operate the device smoothly and correctly. Make sure to read this manual carefully before using this product. Please note that specification and information are subject to changes without prior notice in this manual. Any change will be integrated in the latest release. The manufacturer assumes no responsibility for any errors or omissions in this document.

1. Summary

Working Based on existing GSM/GPRS network and GPS satellites, this product can locate and monitor any remote targets by SMS or internet.

Specifications

| Content | Specs |
| --- | --- |
| Dim. | 93mm x 77mm x 27mm |
| Weight | 227g |
| Network | GSM/GPRS |
| Band | 850/900/1800/1900Mhz |
| GSM chip | MT6261 |
| GPS chip | Ublox 7020 |
| GPS sensitivity | -159dBm |
| GPS accuracy | 5m |
| Time To First Fix | Cold status 45s /Warm status 35s /Hot status 1s |
| Work voltage | 5V |
| Storage Temp. | -40°C to +85°C |
| Operation Temp. | -20°C to +55°C |
| battery | Chargeable 3.7V 6600mah Li-ion Battery Optional car charge:12-30V |
| Humidity | 5%--95% non-condensing |

# 2.Main functions

- GSM quad-band frequency;

- Real time tracking by SMS/GPRS

- Low battery alarm

- Movement alarm

- Into/out off Geo-fence alarm

- Add Vibration alarm

- Add Monitor

- Add Super-long standby 3 yeas (24-hour cycle positioning）

- Add strong magnet(free installation)

- Add Anti-jammer and Anti-detection function

- Add Anti-Disassemble and anti-Fall off alarm

- Add clock mode /fixed time positioning/smart cycle position

- add waterproof and dust proof

- add save power and GPRS

- Add AGPS function,receive GPS signals faster

- Add built-in storage memory

- Add app GPRS set

3.Led indicator status

| # LED INDICATOR STATUS | # MEANING |
| --- | --- |
| # Green LED Flashing | # Searching GSM signal |
| # Green LED continuously lit | # GSM signal OK |
| # Blue LED continuously Flashing per 3s every | # Searching gps signal |
| # Blue LED continuously lit | # gps signal ok |

3.1 Notices:

1).GPS tracking can be worked outdoor. Must have mobile phone gsm signal and have GPS signal,  It can block GPS satellite signal when there are metal or  building above the GPS tracker

2).Pls make sure GSM 2g SIM card or compatibility 2G GSM network sim card, supports making calls, sending messages, GPRS network.have money,Cancel the PIN code

- Quick using

Use mobile phone call the device and hang up

or sending message “smslink123456” to sim card in tracker ,then send message “time zone123456 time”tracker reply positioning information,time is loca time zone

Picture (1) have gps signal     Picture (2) no gps signal

Pic(1)：Replied real-time tracking information

Lat:22.67533 lon:114.03913  Positioning of the latitude and longitude values.

T:06/14/17 09:14  real-tracking time, the Default is London time. You can set a time zone

Google Link: This is the link to the Google map showing the position of the device.

- Pic( 2)：Replied last time position information. Now is LBS position

If you receive positioning information is picture 2, please go out the outdoor and Walk around with gps tracker,for receive The satellite signal

5:Support Website tracking or APP tracking check

If you need real-time tracking by APP ,please scan APP or Download “dagps”

5.1 Scan APP or Download APP

1)scan QR code---open with browser--- choose Android or Iphone---Download--  installation app

2) Download ios APP:login in app store---dagps---Install the dagps APP

5.2: login platform  positioning

1)Open www.gps110.org

Use tracker ID login, default password 123456.

PC login                        APP login

5.3 register Administrator account and tracking multiple devices

operation ：register APP accounts--- Fill in the APP---login in registered accounts vehicle list----add device---fill in ID--OK

Registration                                        fill in

Click vehicle list                            add device

If you add device more than two , continue to add ID number

If device online in platform, no need set apn commands

6.1 Set APN

SMS1: APN123456+space+Apn name

( APN name pls ask the sim card company ,for example tell you the apn name is ABC,so send :APN123456 ABC)

(If have the APN user name and password, need plus APN user name and password behind the APN name)

Eg: APN123456+space+APN name+space+ APN user name+space+   APNpassword

It is Real time positioning,every 3min upload position by default,you can change upload time interval

6.2. If use the other platform, start on line tracker

in advance Using mobile phone send SMS commands to device and set online platform (please set DNS and port,set apn)

(Use the default platform, don't need to operate 6.2)

7)  Working mode

【Mode 1】：real-time tracking (default)

every 3mins/1 times,can work 10 days,can’t anti-jammer and anti-detection

【Mode 2】：24 hours cycle mode

every 24 hours/1 times positioning,can work 3 years

【Mode 3】：clock mode(can set up 2-6 times positioning per day )

every 24 hours/2-6 times positioning,can work from 6 months to 18 months

【Mode 4】:  smart cycle positioning(10mins)

every 10 mins/1 times, device upload position , then device turn off

Until the next cycle positioning again

Positioning time interval can be changed, time is ≥10mins

【Mode 5】:  smart turn on/off mode (deep sleep save power mode)

(static turn off save power, vibration or falls off alarm start work)

NOTE: mode2、mode3、mode4 and mode5 is saving power mode

- 1)After device upload position to platform ,device will turn off automatic for save power)

- 2)During turn off,device not reply and offline platform is normal

- 3)device will online in 3 mintutes next cycle positioning,you can set device or change device mode at this period

8.Set Command by APP or SMS

NOTE:

1) Device default password:123456)

2)use of mobile phones APP or send message to the tracker about all set command

3)If you want standby time longer,  pls don't open move alarm, and real-time  location tracking.etc

4)Charging for 24 hours for the first time, device light not on

| NAME | COMMAND | - EXAMPLE |  |
| --- | --- | --- | --- |
| Set apn | apn123456 apn Name |  | apn123456 cmnet |
|  | apn123456 apn name user password |  | Apn123456 cmnet cmwap 66666 |
| Set IP /DNS port | adminip123456 IP port |  | Adminip123456 110.92.64.21 7018 |
|  | Adminip123456 DNS port |  | Adminip123456 www.gps110.org 7018 |

| INSTRUCTION | COMMAND | REPLY |
| --- | --- | --- |
| WHERE | smslink123456 | Get adress |
| Set admin number | admin123456 mobile number | admin ok |
| Cancel admin number | noadmin123456 mobile number | noadmin ok |
| Set time zone | time zone123456 8 | Time zone ok |
| Set time zone(half time zone) | Time zone123456 8 30 | time zone ok |
| set move alarm | move123456 (suggest not open for will not save power | move ok |
| Cancel move alarm | nomove123456 | nomove ok |
| Open low battery alarm | Lowbattery123456 on | Low batter ok |
| Cancel low battery alarm | Lowbattery123456 off | Low batter ok |
| Cancel GPRS | sms123456 | smsok |
| Set shock alarm | Shock123456 | Shock ok |
| Cancel shock alarm | noshock123456 | noshock ok |
| Listening to the sound | monitor123456 | Monitor ok |
| Cancel listening sound | tracker123456 | trackerok |
| Clock mode | clock123456 1200 1800 2100 (the time format is 24 hours system) | Clock ok |
| Smart turn on/off mode | static123456 | Static ok |
| Smart cycle mode(≥10mins) | Time123456 10 | time ok |
| real-time tracking (default) | t003m***n123456 | T n ok |
| Fall off alarm | Loosing123456 | Loosing ok |
| Check param | check123456 | param |
| Reset factory | reset123456 | reset ok |

| FAQ | Instructions/Solutions |
| --- | --- |
| Fail to turn it on | Please check device charged |
| No GSM signal | - Please check if SIM card installed correctly. - Please check if SIM card is 2G GSM network. - Don’t turn on the PIN code - no calls can be diverted - Please check if voltage of the power is normal - Please check if voltage of the power is normal 电源电压是否正常 |
| No GPS | A more open view of the sky is recommended to make sure the device can receive the GPS signal normally.in outdoor,or take device to walk around |
| No reply to SMS command | SMS commands password wrong or the format is wrong. restore factory settings send “begin123456” |
| No reply to calling & No alarm message | Authorized / admin phone number is incorrect or has not set up,check sim card is normal |
| SMS position OK, cannot use the APP and web platform | Pls check if APN is correctly set. And pls find out the correct APN content of the SIM card. |
| Platform position is difference from the actual position. | No uploading position time or no GPS signal,reply position is last times. If it shows like Pic 2, pls make sure the device can receive the GPS signal normally.in outdoor,and take device to walk around |